| ## Purpose: | ## Is to ensure the safety of GWMWater employees and the general public when crossing roads with the dozer/log skidder and ditching equipment and to protect road and rail infrastructure. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ## Competencies and Training Required |  | - Training in this procedure - Basic Worksite Traffic/Traffic Controller accredited training - Plant Qualification for the Dozer Operator |  |  |  |
| Personal Protective Equipment / Clothing Required |  |  | Additional PPE/C Required During summer appropriate PPE must be worn including a hat and sunscreen. |  |  |
| ## Preparation The following procedures are to be followed dependent on the type of road and the amount of traffic that utilises it. - Main Roads with greater than 200 cars passing in a day When crossing main roads that have a consistent traffic flow, two “Prepare to Stop” signs will be placed not closer than 2D metres (where D is the speed limit) either side of the crossing point. Two traffic controllers will be positioned not closer than 50 metres either side of the crossing point. The ditcher gate opener will not be a traffic controller. - Main Roads with less than 200 cars passing in a day When crossing main roads where traffic is infrequent, two “Prepare to Stop” signs will be placed not closer than 2D metres (where D is the speed limit) either side of the crossing point. One traffic controller will be placed at the crossing point. The ditcher gate opener may be the traffic controller. |  |  | - Minor Roads (Unsealed) When crossing unsealed roads that generally have low traffic flows, one traffic controller will be placed at the crossing point. The ditcher gate opener may be the traffic controller. - Rail Crossings Ditching operations must only cross railways at designated rail crossings. Under no circumstances shall the dozer/log skidder and the ditching equipment cross over exposed tracks. A train timetable should be obtained to ensure trains are not scheduled at the same time as crossing the tracks with the Dozer. - Special Conditions There are certain conditions that operators must consider when choosing the right procedure to follow. These include, blind hills and corners, fog, rain, dust storms etc. When these conditions prevail the main roads with greater than 200 cars passing in a day must be followed. Signs can be placed further than 2D if required but additional signage may be required. | ## Operation - Cross the road following the appropriate process outlined in step 1 Preparation. |  |
|  |  |  |  | ## Finishing / Clean Up - Pack away all equipment including signs, rubber mats etc immediately after the plant has finished crossing the road. Ensure you remain constantly aware of traffic while completing this task. |  |

Do’s and Don’ts

Always ask someone if you are unsure. It is better to ask than to suffer a work injury or sustain damage to plant and equipment.

Never use plant and equipment that is faulty or damaged. Always report this to your supervisor.

Always report faulty plant and equipment to your supervisor. Not reporting can endanger the safety of others.

Never take short cuts and always follow the Safe Work Instruction and Operating Manuals.