| ## Purpose: | ## To cover the operation of flying a small Drone (remotely piloted aircraft) weighing under two kilograms safely |
| --- | --- |

| ## Competencies and Training Required | - Training in this procedure - Read the Operation Manual - Complete an in-house Competency Based Assessment |  |  |  | ### Insert Photo and/or Additional Comments Here ### (if applicable) |
| --- | --- | --- | --- | --- | --- |
| Personal Protective Equipment / Clothing Required Only include those symbols as required |  |  | Additional PPE/C Required -Hi Vis Safety vest -Sunscreen & wide brimmed hat in summer months |  |  |
| ## Preparation - Notify CASA and submit an RPAS notification application and wait at least 5 days before first planned flight and apply/obtain for ARN (Aviation reference number). This notification is valid only for 3 years and then must be re-applied for. - Drone equipment and controller batteries suitably charged - Check flight path for obstacles, such as powerlines, trees and surrounds and plan to avoid - Complete Drone flight JSA |  | - Operation - Turn Controller on first - Turn Drone on second - Check Drone and ensure back lights have turned onto green and hooked up to satellite - Commence drone flying operation and follow CASA standard operating procedures, which includes; Only fly during daylight hours and keep Drone within visual sight, Don’t fly Drone above 120 metres, Keep Drone at all times at least 30 metres away from other people, Keep Drone at least 5.5 Km away from controlled aerodromes, Use app Can I fly there, Don’t fly Drone over any populous area, including beaches, parks and sporting ovals, Don’t fly Drone over an area affecting public safety or where emergency operations are underway, such as car crash, fire or search & rescue and Only fly one Drone at a time. - Follow Drone flight path on monitor and also visually at all times - Operate Camera once drone is in position over asset - Continually review and monitor the Drone battery light to ensure it maintains charge |  | - Finishing / Clean Up - If Drone battery light located at the rear turns from Green to red and starts flashing, commence returning Drone to the ground for immediate landing. - Land Drone on the ground and turn Drone off First - Once Drone is turned off, then also turn off Controller - Take battery out of Drone and Controller and replace ready for the next flight |  |

Do’s and Don’ts

- Always ask someone if you are unsure. It is better to ask than to suffer a work injury or sustain damage to equipment.

- Never use equipment that is faulty or damaged. Always report this to your supervisor.

- Always report faulty equipment to your supervisor Not reporting faulty or damaged equipment can endanger the safety of others.

- Never take short cuts and always follow the Safe Work Instruction and Operating Manual.